D-500.4 Firmware Update Steps

1, Connect the computer and equipment with a USB data cable.

2, Open the DSP218 FirmwareUpdate program software, as shown below:

①: Select baud rate, the default value is 57600.

②: Select the Comport port number, access to computer port number as follows:

Note:View the port number in Computer Management

③: Click Connect, Status light turns green.

④: Click Firmware Browser, select the .bin file which need to update, as shown below:

⑤: Select Normal Program in Update Progress.

Note: The Bad Rom Program can only be selected if the Select Bad Rom Program appears on 	the device display.

⑥: Click Program to update.

⑦: During the update process, the progress bar will show the progress, when updated to 100%, said 	the end of the update.

3, After the completion of the update, waiting for the device to automatically restart. In the device 	automatically restart process, can not power down and key operation.